Shore Power Grounding Investigation

Quick Reference Guide

Get history, if possible, from owner/operator

Start by exploring the system:

Safety

- You’ll be working on the AC system

- Use voltage detector

- PPE, ear & eye

Gather all tools

- Tool bag, Socket set, Multi-meter,

- AC Split Cord, AC leak Detector?

Start by plugging in an AC device ​in a non-inverted outlet

Note:*if entire boat goes through inverter, you need to perform these test on the input side of the inverter

- Using an AC appliance or electrical tester, we are verifying power is working onboard

- With some load turned on in the vessel, test for AC leakage, report if found.

- Make sure the outlet works​ and has correct polarity

- *Do not, turn off, the main shore breaker onboard the vessel​

Shut off shore power:

- Turn off the breaker at pylon​

- Bring the power pylon end into vessel at outlet or inverter input.

Test#1 Check the ground circuit with DVOM​

- Check ground pin on outlet to ground on cord.​

  - If “0” ohms resistance (or very low)​

    - Vessel is safe, but doesn’t have galvanic protection​

    - Recommend installing a galvanic isolator​:

    - Skip to Test#3 and check for a transformer

Test#2 the ground circuit with DVOM checking for Galvanic Isolator

- Check ground pin on outlet to ground on cord.​

  - Run the Test procedure for a Galvanic Isolator​

  - *Remember, could have capacitors​

  - Report findings of G.I. on report. Stop test here

Test#3 the ground circuit with DVOM checking for Isolation Transformer

- Check ground pin on outlet to ground on cord.​

  - If “OL” on resistance, an open line​   Verify findings​

  - Verify No Galvanic Isolator​

  - Ensure you are on the ground pin of cord​

- Possibly:​

  - Isolation Transformer​

  - Or Un-safe vessel​

  - Ensure you are on the ground pin of cord​

- Check for transformer​

- *main breaker must be on!​

- Check the hot/neutral wires for connectivity ​

- If the hot/neutral wires don’t have path an “OL”​, Then you know it has transformer because you had power onboard when starting this test​

- Possibly locate the transformer and give it basic inspection

If “OL” resistance, an open line​ on Green, but not the Hot/Neutral

- Verify findings​

- Verify: No Galvanic Isolator​

- Verify: No transformer [hot/neutral wire open]​

- Verify: No connectivity on green wire [OL]​

- Report: Unsafe vessel, must report​

Tech Tip#74 Video about testing shore power isolation with Nigel Calder

​